✅ Your Current Setup Summary

🔹 VNets and IPs:

- Spoke VNet: 192.168.0.0/29

  - Test-VM: 192.168.0.4

- Hub VNet: 10.128.1.0/29

  - VPN Gateway is here

- Firewall VNet: 10.78.0.0/29

  - Desti-VM: 10.78.0.5

🔹 VNet Peerings:

- Spoke ↔ Hub ✅

- Firewall ↔ Hub ✅

🔁 No direct peering between Spoke and Firewall → traffic must go through Hub VNet and VPN Gateway

✅ User-Defined Routes (UDRs):

🟢 In Spoke VNet:

- Route to 10.78.0.0/29 → Next Hop: Virtual Network Gateway

- Associated to: Test-VM's subnet ✅

🟢 In Firewall VNet:

- Route to 192.168.0.0/29 → Next Hop: Virtual Network Gateway

- Associated to: Desti-VM's subnet ✅

❌ No UDR associated to GatewaySubnet (Correct) ✅

🔁 BGP Status

- BGP is enabled ✅
(Needed so that route propagation happens between VPN Gateway and connected VNets)

❌ Ping Still Not Working — Likely Causes

1. 🔒 NSG Rules Blocking ICMP

- Confirm NSGs on both VMs' subnets and NICs allow:

  - Inbound ICMP (Echo Request)

  - Outbound ICMP (Echo Reply)

💡 Use “Effective Security Rules” in the VM's Networking tab to check actual rules.

2. 🔀 Return Route Not Flowing Correctly

- Use Effective Routes from both VMs to verify:

  - Route from Spoke VM → 10.78.0.0/29 → via Virtual Network Gateway ✅

  - Route from Firewall VM → 192.168.0.0/29 → via Virtual Network Gateway ✅

💡 If any route is showing “None” or going to “Internet”, it’s misconfigured.

3. 🔄 Hub VPN Gateway Configuration

- Is the VPN Gateway connected to another network (e.g., on-prem or ExpressRoute)?

  - If not, then the Virtual Network Gateway as next hop may not forward within Azure — this is critical:

🚫 VPN Gateway does not forward traffic between two VNets using it as a hub unless it's connected externally or is a Route Server or NVA.

⚠️ Important Azure Limitation

If you're using only Azure VPN Gateway without connecting it to on-premises and just want to use it as a router between VNets, it won't work.

Azure VPN Gateway is not designed to route VNet-to-VNet traffic unless via actual VPN/ER connections.

✅ Recommended Solutions

Option 1: 🔀 Use Azure Route Server + NVA

- Deploy Azure Route Server in the Hub VNet

- Add a Network Virtual Appliance (NVA) (e.g., a VM with routing)

- Configure UDRs to route via the NVA

Option 2: 🛣️ Use Full Mesh Peering

- Peer Spoke ↔ Firewall VNet directly

- Allow gateway transit via Hub only if needed

✅ Immediate Test Actions

- Check NSGs on both VMs – ICMP must be allowed inbound and outbound

- Check Effective Routes on both VMs

- Verify peering is configured with “Use remote gateways” and “Allow forwarded traffic” enabled on Spoke and Firewall peerings

- If nothing external is connected to VPN Gateway, this architecture won’t support routing traffic across VNets via VPN Gateway only

Would you like help redesigning it (e.g., using NVA or Azure Route Server)?

You said:

So VPN gateway it wont work??? We need Azure Firewall or Azure route server???

ChatGPT said:

Great question — and yes, this is a common point of confusion.

🚫 Why VPN Gateway Alone Won’t Work for Intra-VNet Routing

Azure VPN Gateway is not designed to route traffic between VNets unless:

- It's connected to on-prem or another network via Site-to-Site VPN, ExpressRoute, etc.

- It's used as a gateway for hybrid connectivity, not for routing between VNets.

So in your case — using a VPN Gateway in Hub VNet to route traffic from Spoke to Firewall VNet — it won’t work on its own.

✅ What You Actually Need (Alternatives)

You need something in the Hub that can receive and forward traffic between VNets. Azure gives you 2 main options:

✅ Option 1: Azure Route Server + NVA (Recommended for advanced routing)

- Deploy Azure Route Server in Hub VNet

- Add a Network Virtual Appliance (NVA) like a Cisco/CheckPoint/Ubuntu VM with IP forwarding enabled

- Configure:

  - BGP between Route Server and NVA

  - UDRs in Spoke/Firewall VNet to send traffic to the NVA

🧠 Use this if you need dynamic routing, security inspection, or advanced control.

✅ Option 2: Azure Firewall in Hub VNet

- Deploy Azure Firewall in the Hub VNet

- Route:

  - Spoke → Hub (Azure Firewall)

  - Hub → Firewall VNet

🔥 Azure Firewall supports traffic inspection and routing, unlike VPN Gateway.

✅ Option 3: Full Mesh Peering

If your goal is just to connect Spoke and Firewall VNet, and no inspection/gateway is needed:

- Simply peer Spoke ↔ Firewall VNet directly

- In peering settings:

  - Enable "Allow forwarded traffic"

  - Enable "Use remote gateway" if you still need VPN gateway transit for other traffic

This is the simplest solution if you don't need inspection or centralized control.

✅ Summary

| Solution | Can Route Between VNets? | Notes |
| --- | --- | --- |
| VPN Gateway | ❌ Not internally | Only works for hybrid connectivity |
| Azure Firewall | ✅ Yes | Good for centralized routing + security |
| Route Server + NVA | ✅ Yes | Best for complex routing via BGP |
| Full Mesh Peering | ✅ Yes | Easiest, but less control |

🧭 What should you do?

If you don't need security inspection:

- Use VNet peering between Spoke and Firewall VNets directly

If you want centralized routing/security:

- Use Azure Firewall or Route Server + NVA in the Hub